Reinstall Raspberry Pi software via PI imager

Making sure SPI & I2C are active:

sudo raspi-config (-> P3; interface options)

activate: SPI & I2C sudo reboot

Update all software:

sudo apt update

sudo apt upgrade

sudo apt install python3-venv python3-pip python3-smbus i2c-tools python3-dev build-essential

Make a direction where all the needed packages will be installed:

mkdir ~/testing

cd ~/testing

python3 -m venv env

source env/bin/activate

Install all the packages:

python3 -m pip install spidev

pip install pygame

pip install pyrf24

sudo apt install python3-rpi-lgpio

sudo apt install build-essential python3-dev swig liblgpio-dev

(in env) pip install rpi-lgpio

pip install --no-deps adafruit-blinka adafruit-platformdetect adafruit-pureio

pip install --no-deps adafruit-circuitpython-servokit \
  adafruit-circuitpython-pca9685 \
  adafruit-circuitpython-motor \
  adafruit-circuitpython-busdevice \
  adafruit-circuitpython-register \
  Adafruit-circuitpython-typing

pip install typing-extensions --no-deps

Connecting GitHub:

sudo apt install -y gh

gh auth login

Go to git website (via command screen) and log in to get the code

Fill in the code in the command screen and then

git clone https://github.com/TijgervdD/leafy.git

Note: because it is a raspberry pi 5 the adafruit servomotor HAT software hass to be downloaded through such a way, otherwise you get a GPIO error. This is because of the software differences between pi 5 and younger Pi’s